[SoC Design]

Week 6 
(from github repo)

Report 6

Student Number: 2023066980

Student Name: 김영진

Explain the waveform results along with the implemented “to-do” code, including the operation of the testbench.

- Include the captured “to-do” code.

- Include the captured waveform.

- Write the report in English.

Lab1

- Simulation with the pre-compiled code (Capture Full Screen)

- Simulation Passed Code MDK Keil + test.c code (Capture Full Screen, Only one)

- FPGA Board Demo

a)

// --=========================================================================-- // Copyright (c) 2025-2026 DSAL, Hanyang University. All rights reserved // DSAL Confidential Proprietary // ---------------------------------------------------------------------------- // This confidential and proprietary software may be used only as // authorised by a licensing agreement from DSAL. // ----------------------------------------------------------------------------- // FILE NAME : hy_soc.v // DEPARTMENT : Digital System Architecture Lab. // AUTHOR : Ji-Hoon Kim // AUTHOR'S EMAIL : jhoonkim@hanyang.ac.kr // ----------------------------------------------------------------------------- // hy_soc.v — HY-SoC Educational Lab1 // // Cortex-M0 (DesignStart) + AHB-Lite Shared Bus + SRAM 64KB + Default Slave // // Memory map: // 0x0000_0000 – 0x00FF_FFFF : s0 — ahb_sram (64 KB actual, 16 MB window) // other : nomap → ahb_default_slv (ERROR response) module hy_soc ( input wire clk, // system clock //input wire rst_n, // async assert, sync deassert (active-low) input wire i_rst, output wire [3:0] led ); wire rst_n; assign rst_n = !i_rst; // --------------------------------------------------------------------------- // AHB-Lite bus signals // --------------------------------------------------------------------------- wire [31:0] haddr; wire [31:0] hwdata; wire hwrite; wire [1:0] htrans; wire [2:0] hburst; wire hmastlock; wire [3:0] hprot; wire [2:0] hsize; wire [31:0] hrdata; wire hresp; wire hready; // Subordinate signals wire hsel_s0; wire [31:0] hrdata_s0; wire hreadyout_s0; // No interrupts in v02 wire [31:0] irq = 32'h0; // --------------------------------------------------------------------------- // Reset synchronizer: two independent async-assert / sync-deassert chains // por_sync (ext reset only) -> PORESETn, DBGRESETn // sys_sync (ext reset + SYSRESETREQ) -> HRESETn // --------------------------------------------------------------------------- wire hresetn, poresetn, dbgresetn; cm0_rst_sync u_rst_sync ( .clk (clk), .rst_n (rst_n), .sysresetreq_i (1'b0), .poresetn_o (poresetn), .dbgresetn_o (dbgresetn), .hresetn_o (hresetn) ); // CDBGPWRUP self-ack wire cdbgpwrupreq; // --------------------------------------------------------------------------- // Cortex-M0 DesignStart // --------------------------------------------------------------------------- CORTEXM0INTEGRATION u_cm0 ( // clocks & resets .FCLK (clk), .SCLK (clk), .HCLK (clk), .DCLK (clk), .PORESETn (poresetn), .DBGRESETn (dbgresetn), .HRESETn (hresetn), .SWCLKTCK (1'b0), .nTRST (1'b1), // AHB-Lite manager port .HADDR (haddr), .HBURST (hburst), .HMASTLOCK (hmastlock), .HPROT (hprot), .HSIZE (hsize), .HTRANS (htrans), .HWDATA (hwdata), .HWRITE (hwrite), .HRDATA (hrdata), .HREADY (hready), .HRESP (hresp), .HMASTER (), // code sequentiality (unused) .CODENSEQ (), .CODEHINTDE (), .SPECHTRANS (), // debug — SWD tied off .SWDITMS (1'b0), .TDI (1'b0), .SWDO (), .SWDOEN (), .TDO (), .nTDOEN (), .DBGRESTART (1'b0), .DBGRESTARTED (), .EDBGRQ (1'b0), .HALTED (), // interrupts .NMI (1'b0), .IRQ (irq), .TXEV (), .RXEV (1'b0), .LOCKUP (), .SYSRESETREQ (), // SysTick: 50 MHz → 10 ms = 500 000 - 1 = 0x07_A11F .STCALIB ({1'b1, 1'b0, 24'h07A11F}), .STCLKEN (1'b0), .IRQLATENCY (8'h00), .ECOREVNUM (28'h0), // power management — tied off .GATEHCLK (), .SLEEPING (), .SLEEPDEEP (), .WAKEUP (), .WICSENSE (), .SLEEPHOLDREQn (1'b1), .SLEEPHOLDACKn (), .WICENREQ (1'b0), .WICENACK (), .CDBGPWRUPREQ (cdbgpwrupreq), .CDBGPWRUPACK (cdbgpwrupreq), // self-ack // scan — tied off .SE (1'b0), .RSTBYPASS (1'b0) ); // --------------------------------------------------------------------------- // On-chip Interconnect (SRAM + default slave) // --------------------------------------------------------------------------- ahb_interconnect u_interconnect ( .hclk (clk), .hrst_n (hresetn), .haddr_i (haddr), .htrans_i (htrans), .hrdata_o (hrdata), .hready_o (hready), .hresp_o (hresp), .hsel_sram_o (hsel_s0), .hrdata_sram_i (hrdata_s0), .hreadyout_sram_i(hreadyout_s0) ); // --------------------------------------------------------------------------- // s0: ahb_sram 64 KB (MEMWIDTH=16 → 2^16 bytes) // --------------------------------------------------------------------------- ahb_sram #(.MEMWIDTH(16)) u_sram ( .hsel (hsel_s0), .clk (clk), .rst_n (hresetn), .hready (hready), .haddr (haddr), .htrans (htrans), .hwrite (hwrite), .hsize (hsize), .hwdata (hwdata), .hreadyout (hreadyout_s0), .hrdata (hrdata_s0) ); localparam DONE_ADDR = 32'h00007FFC; localparam PASS_CODE = 32'h900DD00D; localparam FAIL_CODE = 32'hDEADDEAD; reg mon_pending; reg [31:0] mon_addr; always @(posedge clk or negedge rst_n) begin if (!rst_n) begin mon_pending <= 1'b0; mon_addr <= 32'h0; end else if (hready) begin mon_pending <= hsel_s0 && hwrite && htrans[1]; mon_addr <= haddr; end end // state reg [3:0] c_led, n_led; // sequential always @(posedge clk or negedge rst_n) begin if (!rst_n) c_led <= 4'b0000; else c_led <= n_led; end // combinational keyword = {DONE_ADDR, PASS_CODE, FAIL_CODE, hwdata, mon_addr, mon_pending} always @(*) begin n_led = c_led; // default (중요) if ( mon_pending && mon_addr == DONE_ADDR ) begin if ( hwdata == PASS_CODE ) n_led = 4'b1111; else if ( hwdata == FAIL_CODE ) n_led = 4'b0101; end end assign led = c_led; endmodule

b)

c)

1. Warning Resolution

The eabi_attribute 25, 1 directive was added to startup_cm0.S to suppress existing build warnings.

2. Monitor Logic Description

The monitor logic is designed to observe write access to the ahb_sram. It triggers specifically when the target address matches the debugging address, defined as DONE_ADDR.

Specifically, the logic does not operate by performing a read access to DONE_ADDR. Instead, it monitors the write phase; at the exact moment a write operation is asserted to DONE_ADDR, the logic inspects the hwdata to determine the PASS or FAIL status of the simulation.

3. Comparison with Week 5 Lab

In the previous week's RTL implementation, only the SRAM was connected to the AHB bus as a subordinate. The design was configured to perform write operations unconditionally, without accounting for the master’s specific request states.

However, in the current revision, the interconnect has been modified to manage the ready signal and control write access to the slave channels. This architecture enhances system stability by arbitrating signals such as hready and hdata through the slv_mux module. While Lab 1 continues to utilize a single slave, this implementation is critical for preventing race conditions in Lab 2 and beyond, where multiple slaves are involved.

Lab2

1. Build Optimization

Similar to the previous update, build warnings were suppressed by adding the eabi_attribute 25, 1 directive to startup_cm0.S.

2. LED Control Logic and Configuration

In Test.c, the PASS_BLINK_COUNT was adjusted to 20.

- Blink Definition: For this implementation, a single "blink" is defined as a complete On → Off → On cycle.

- Alternative Definition: If a "blink" were defined as a single state transition (either On → Off or Off → On), a value of 10 would be appropriate.

3. ISR Implementation Details

This configuration is dictated by the led_state ^= 0x0Fu; statement within the SysTick ISR. Since the bitwise XOR (^) operation is executed every time the interrupt handler is invoked, it toggles the 4-bit LED state between its active and inactive phases. The total number of these inversions is controlled by the PASS_BLINK_COUNT value, ensuring the desired visual feedback duration.

In addition, the interconnect architecture has been updated relative to the earlier version of the code.

1. Interconnect and Arbitration Logic

In the previous implementation, the logic was limited to supporting only the SRAM. However, the addition of multiple slaves necessitated an arbitration mechanism to manage channel access rights. Consequently, multiplexing (MUX) logic was integrated to handle the data paths for both the LED and SRAM subordinates.

2. Implementation and Debugging

During the instantiation phase, a discrepancy in the pin naming convention was identified and corrected to ensure proper signal mapping.

3. Verification Strategy

The firmware developed in Keil is configured with a 1-second blink interval. Since simulating such a long period at the RTL/Gate level is computationally expensive and impractical, the functional verification was instead performed through FPGA prototyping.